#include <AFMotor.h>

#include <Servo.h>

// 1. Initialize all 4 DC Motor Ports on the L293D Shield

AF_DCMotor motorFrontLeft(1);  // Connected to M1 screw terminal

AF_DCMotor motorRearLeft(2);   // Connected to M2 screw terminal

AF_DCMotor motorFrontRight(3); // Connected to M3 screw terminal

AF_DCMotor motorRearRight(4);  // Connected to M4 screw terminal

Servo radarServo; // Create servo object to control scanning mount

// 2. Hardware Pin Setup (Mapping Analog Pins as Digital Inputs/Outputs)

const int PIN_TRIG = A0; // Ultrasonic Transmitter

const int PIN_ECHO = A1; // Ultrasonic Receiver

const int PIN_PIR  = A2; // PIR Motion Sensor Digital Pin

const int PIN_GAS  = A3; // MQ Gas Sensor Digital Pin (D0)

const int PIN_LED  = A4; // Red Alert LED Positive Row

// 3. Operational Constants

const int DISTANCE_THRESHOLD = 25;  // Stop and scan if obstacle is closer than 25cm

const int MOTOR_MAX_SPEED    = 255; // Core speed limit (0 to 255 max power)

void setup() {

// Initialize Serial Communication at 9600 Baud

Serial.begin(9600);

Serial.println("==========================================");

Serial.println("   Initializing Smart Obstacle Car Core   ");

Serial.println("==========================================");

// Configure Sensor inputs and Alert outputs

pinMode(PIN_TRIG, OUTPUT);

pinMode(PIN_ECHO, INPUT);

pinMode(PIN_PIR, INPUT);

pinMode(PIN_GAS, INPUT);

pinMode(PIN_LED, OUTPUT);

// Attach the servo motor to the dedicated SERVO_1 port on the L293D shield (Pin 10)

Serial.println("[SYSTEM] Attaching Servo to Pin 10...");

radarServo.attach(10);

radarServo.write(90);  // Look straight ahead initially

// Apply uniform driving speed to all wheel channels

Serial.println("[SYSTEM] Setting motor speeds to maximum power...");

updateCarSpeed(MOTOR_MAX_SPEED);

Serial.println("[SYSTEM] Setup complete! System running.");

Serial.println("------------------------------------------");

delay(1000);

}

void loop() {

// Task 1: Check safety critical sensors (Gas & Motion)

bool hazardFound = monitorSafetySensors();

// Task 2: Calculate path clearance ahead

int forwardClearance = measureDistancecm();

// Print real-time status to Serial Monitor

Serial.print("[RADAR] Forward Distance: ");

Serial.print(forwardClearance);

Serial.println(" cm");

// Task 3: Autonavigation Decision Matrix

if (forwardClearance > 0 && forwardClearance < DISTANCE_THRESHOLD) {

Serial.println("[ALERT] OBSTACLE DETECTED! Stopping car...");

haltAllWheels();

delay(300);

// Look around using the servo radar mount

Serial.println("[RADAR] Executing scanning sweep...");

int rightClearance = scanEnvironmentAngle(30);   // Sweep Right

Serial.print("[RADAR] Right Scan Result: "); Serial.print(rightClearance); Serial.println(" cm");

int leftClearance  = scanEnvironmentAngle(150);  // Sweep Left

Serial.print("[RADAR] Left Scan Result: "); Serial.print(leftClearance); Serial.println(" cm");

radarServo.write(90); // Reset radar frame back to center

delay(300);

// Evaluate open spaces

if (leftClearance > rightClearance && leftClearance > DISTANCE_THRESHOLD) {

Serial.println("[NAV] Left side is clearer. Turning Left.");

executeLeftTurn();

delay(600); // Duration of rotation

}

else if (rightClearance > leftClearance && rightClearance > DISTANCE_THRESHOLD) {

Serial.println("[NAV] Right side is clearer. Turning Right.");

executeRightTurn();

delay(600);

}

else {

// Both sides are blocked or clear layout unknown

Serial.println("[NAV] Both paths blocked. Reversing and searching alternative path.");

executeReverse();

delay(800);

executeRightTurn();

delay(500);

}

haltAllWheels(); // Brief stop before committing back to forward stride

delay(200);

}

else {

// Road clear and no local barrier: Proceed forward safely

driveForward();

}

delay(100); // Small delay to prevent flooding the Serial Monitor window

}

// ==========================================

//        RADAR & PULSE SYSTEM FUNCTIONS

// ==========================================

int scanEnvironmentAngle(int angleValue) {

radarServo.write(angleValue);

delay(450); // Give the SG90 motor gears physical time to arrive at location

return measureDistancecm();

}

int measureDistancecm() {

digitalWrite(PIN_TRIG, LOW);

delayMicroseconds(2);

digitalWrite(PIN_TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(PIN_TRIG, LOW);

// Reads acoustic rebound with a 25ms maximum ceiling boundary timeout

long soundDuration = pulseIn(PIN_ECHO, HIGH, 25000);

if (soundDuration == 0) {

return 400; // Out of frame bounds or clear infinity path (Default to 4 meter max space)

}

// Math: (Duration / 2) divided by speed of sound standard metric coefficient (29.1)

return (soundDuration / 2) / 29.1;

}

// ==========================================

//        SAFETY & NOTIFICATION DRIVERS

// ==========================================

bool monitorSafetySensors() {

int gasStatus = digitalRead(PIN_GAS);

int pirStatus = digitalRead(PIN_PIR);

bool activeHazard = false;

if (gasStatus == HIGH) {

Serial.println("[HAZARD] ** WARNING: Toxic Gas Leak Detected! **");

activeHazard = true;

}

if (pirStatus == HIGH) {

Serial.println("[HAZARD] ** WARNING: Motion Intrusion Detected! **");

activeHazard = true;

}

if (activeHazard) {

digitalWrite(PIN_LED, HIGH);

return true;

} else {

digitalWrite(PIN_LED, LOW);

return false;

}

}

// ==========================================

//        H-BRIDGE WHEEL MOTION SYSTEM

// ==========================================

void updateCarSpeed(int speedValue) {

motorFrontLeft.setSpeed(speedValue);

motorRearLeft.setSpeed(speedValue);

motorFrontRight.setSpeed(speedValue);

motorRearRight.setSpeed(speedValue);

}

void driveForward() {

motorFrontLeft.run(FORWARD);

motorRearLeft.run(FORWARD);

motorFrontRight.run(FORWARD);

motorRearRight.run(FORWARD);

}

void executeReverse() {

motorFrontLeft.run(BACKWARD);

motorRearLeft.run(BACKWARD);

motorFrontRight.run(BACKWARD);

motorRearRight.run(BACKWARD);

}

void executeRightTurn() {

motorFrontLeft.run(FORWARD);

motorRearLeft.run(FORWARD);

motorFrontRight.run(BACKWARD);

motorRearRight.run(BACKWARD);

}

void executeLeftTurn() {

motorFrontLeft.run(BACKWARD);

motorRearLeft.run(BACKWARD);

motorFrontRight.run(FORWARD);

motorRearRight.run(FORWARD);

}

void haltAllWheels() {

motorFrontLeft.run(RELEASE);

motorRearLeft.run(RELEASE);

motorFrontRight.run(RELEASE);

motorRearRight.run(RELEASE);

}